## Pye Unicam Atomic Absorption Spectrophotometry

## Flame Analysis, Fuel-oxidant mixtures

# Atomic number, Element

13	Aluminium	Al	N2O/C2H2	fuel rich

51	Antimony	Sb	Air/C2H2	stoichiometric

33	Arsenic	As	N2O/C2H2, Ar/H2	fuel lean

56	Barium	Ba	N2O/C2H2	stoichiometric

4	Beryllium	Be	N2O/C2H2	fuel rich

83	Bismuth	Bi	Air/C2H2	fuel lean

5	Boron	B	N2O/C2H2	fuel rich

48	Cadmium	Cd	Air/C2H2	fuel lean

55	Caesium	Cs	Air/C2H2	fuel lean

20	Calcium	Ca	N2O/C2H2, Air/C2H2	stoichiometric/ fuel rich

24	Chromium	Cr	N2O/C2H2, Air/C2H2	fuel rich

27	Cobalt	Co	Air/C2H2	fuel lean

29	Copper	Cu	Air/C2H2	fuel lean

66	Dysprosium	Dy	N2O/C2H2	fuel rich

68	Erbium	Er	N2O/C2H2	fuel rich

63	Europium	Eu	N2O/C2H2	fuel rich

31	Gallium	Ga	Air/C2H2	fuel lean

32	Germanium	Ge	N2O/C2H2, Air/C2H2	fuel rich

79	Gold	Au	Air/C2H2	fuel lean

72	Hafnium	Hf	N2O/C2H2	fuel rich

67	Holmium	Ho	N2O/C2H2	fuel rich

49	Indium	In	Air/C2H2	fuel lean

77	Iridium	Ir	Air/C2H2	stoichiometric

26	Iron	Fe	Air/C2H2	fuel lean

57	Lanthanium	La	N2O/C2H2	fuel rich

82	Lead	Pb	Air/C2H2	fuel lean

3	Lithium	Li	Air/C2H2	fuel lean

12	Magnesium	Mg	N2O/C2H2, Air/C2H2	stoichiometric

25	Manganese	Mn	Air/C2H2	fuel lean

80	Mercury	Hg	Air/C2H2	fuel lean

42	Molybdenum	Mo	N2O/C2H2, Air/C2H2	fuel rich

60	Neodymium	Nd	N2O/C2H2	fuel rich

28	Nickel	Ni	Air/C2H2	fuel lean

46	Palladium	Pd	Air/C2H2	fuel lean

15	Phosphorus	P	N2O/C2H2	fuel rich

78	Platinum	Pt	Air/C2H2	fuel lean

19	Potassium	K	Air/C2H2	fuel lean

75	Rhenium	Re	N2O/C2H2	fuel rich

45	Rhodium	Rh	N2O/C2H2, Air/C2H2	fuel lean

37	Rubidium	Rb	Air/C2H2	fuel lean

44	Ruthenium	Ru	Air/C2H2	fuel lean

62	Samarium	Sm	N2O/C2H2	fuel rich

34	Selenium	Se	N2O/C2H2, Ar/H2	fuel lean

14	Silicon	Si	N2O/C2H2	fuel rich

47	Silver	Ag	Air/C2H2	fuel lean

11	Sodium	Na	Air/C2H2	fuel lean

38	Strontium	Sr	N2O/C2H2, Air/C2H2	stoichiometric

52	Tellurium	Te	Air/C2H2	fuel lean

81	Thallium	TI	Air/C2H2	stoichiometric

50	Tin	Sn	N2O/C2H2, Air/C2H2	fuel rich

22	Titanium	Ti	N2O/C2H2	fuel rich

23	Vanadium	V	N2O/C2H2	fuel rich

39	Ytterbium	Yb	N2O/C2H2	fuel rich

30	Zinc	Zn	Air/C2H2	fuel lean

The two most commonly used flames are the air/acetylene flame and the nitrous oxide/acetylene flame. The nitrous oxide flame is much hotter than the air-based flame, and is essential for the determination of refractory elements such as Al and Si. The flame temperature of any flame will be affected by the fuel/oxidant ratio, which will also change the oxidising/reducing nature of the flame.

A fuel rich nitrous oxide/acetylene flame has a red cone, the higher the red cone, the richer (= more reducing) the flame. The required height of the red cone is usually specified in the handbooks of standard conditions.

A fuel rich nitrous oxide/acetylene flame has a tendency to form a deposit on the burner. Such a deposit can clog up the burner. It is therefore necessary to work as short a time as possible while using such a flame.

The argon/hydrogen/entrained air flame is transparent.